Property Inspection Report

Prepared by Viewpoint Ark

1. Property & Inspection Details

| Property Information | Inspection Details |
| --- | --- |
| Address: 12 Maple Drive Sandyford, Dublin 18 Co. Dublin, D18 XY22 Property Type: Semi-Detached Dwelling Year Built: Approx. 1992 Gross Floor Area: Approx. 142 m² Number of Storeys: two No. of Bedrooms: four No. of Bathrooms: two | Report Reference: VPA-2025-0412 Inspection Date: 7 April 2025 Inspection Time: 10:00 – 12:30 Inspector: Ciarán Ó'Brien, MRICS Qualification: Chartered Building Surveyor Client Name: Ms. Sarah Murphy Client Reference: SM-2025-047 Report Date: 7 April 2025 |

2. Executive Summary

This report presents the findings of a visual property inspection conducted at the above address on behalf of the client.

The property is a well-presented semi-detached dwelling in satisfactory condition for its age and type. Several items require attention, ranging from routine maintenance to more significant defects as detailed within this report. No structurally critical defects were identified during the inspection; however, further investigation is recommended in certain areas as noted herein.

Condition Rating Key:

| Rating | Description |
| --- | --- |
| 1 – Good | No repair is currently needed. Normal maintenance required. |
| 2 – Fair | Repairs or replacements are needed but not considered urgent. |
| 3 – Poor | These are serious defects that must be repaired or replaced urgently. |
| NI – Not Inspected | This element was not inspected. See limitations noted within the report. |

3. External Elements

| Item | Condition | Notes / Observations |
| --- | --- | --- |
| Roof Covering (Slates/Tiles) | 2 – Fair | Several ridge tiles require re-bedding. Recommend further inspection from roof level. |
| Chimneys & Flashings | 2 – Fair | Minor pointing defects noted to chimney stack. Flashing appears intact. |
| Gutters & Downpipes | 2 – Fair | Blockage evident in rear gutter. Downpipe at rear shows minor rust at bracket. |
| External Walls (Render) | 1 – Good | No significant cracking. Minor hairline cracks to rear elevation — monitor. |
| External Joinery (Fascia/Soffit) | 1 – Good | uPVC fascia and soffit in good condition throughout. |
| Windows & External Doors | 1 – Good | Double-glazed UPVC windows are in good order. Front door draught seal worn. |
| Driveway & Path | 2 – Fair | Block paving shows minor settlement and weed growth in joints. |
| Boundary Walls & Fences | 1 – Good | Garden walls sound. Rear timber fence requires a replacement panel (west side). |
| Drainage (Inspection Chambers) | 2 – Fair | One inspection chamber cover cracked — replacement recommended. |

4. Internal Elements

| Item | Condition | Notes / Observations |
| --- | --- | --- |
| Ceilings | 1 – Good | Plasterboard ceilings are in good condition throughout. Minor hairline crack in hallway. |
| Internal Walls & Partitions | 1 – Good | Dry-lined walls are in good condition. No significant cracking noted. |
| Internal Floors | 2 – Fair | Laminate flooring in sitting room shows localized lifting near patio door. |
| Staircase & Balustrade | 1 – Good | Timber staircase is in good order. Balustrade is secure and compliant. |
| Internal Doors & Frames | 1 – Good | All doors close and latch correctly. No significant defects noted. |
| Fireplace & Chimney Breast | 2 – Fair | Chimney breast has been removed at first floor — structural beam to be confirmed. |
| Loft / Roof Space | 2 – Fair | Insulation present but below current recommended depth (270mm). No active leaks. |
| Damp & Moisture | 2 – Fair | Elevated moisture readings recorded at base of rear kitchen wall — investigation advised. |
| Ventilation | 1 – Good | Trickle vents present to windows. Extractor fans operational in kitchen and bathrooms. |

5. Services & Installations

Note: Services are inspected visually only. Full testing by a qualified engineer is recommended prior to purchase.

| Item | Status | Comments |
| --- | --- | --- |
| Mains electricity supply present and connected | Yes | Consumer unit (fuse board) updated approx. 2018 per vendor. |
| Evidence of rewiring visible | Partial | Rewiring evident on the ground floor; first floor wiring appears older. |
| Gas supply present | Yes | Gas boiler — Vaillant EcoTec. Annual service recommended. |
| Central heating operational | Yes | System tested during inspection. All radiators heated evenly. |
| Hot water system operational | Yes | Mains pressure system. Hot water delivered within acceptable time. |
| Drainage — all outlets flowing freely | Yes | All sinks, baths and WCs flushed and observed draining freely. |
| Evidence of CCTV drainage survey available | No | Recommend commissioning a CCTV drain survey prior to purchase. |
| Smoke/CO detectors present | Partial | Smoke detectors are present on each floor. CO detector absent — recommend installation. |
| Broadband/telecommunications | Yes | Fiber connection available to the property. |

6. Energy Performance

| BER Rating | BER Number | BER Assessment Date |
| --- | --- | --- |
| C2 | 123456789 | 15 March 2025 |

Observations: The property has a C2 BER rating. Improving attic insulation depth and replacing the gas boiler with a heat pump would be the most cost-effective upgrades to improve the rating. The Sustainable Energy Authority of Ireland (SEAI) may offer applicable grant support.

7. Summary of Recommended Actions

| Item | Priority | Est. Cost (€) | Who to Engage |
| --- | --- | --- | --- |
| Re-bed ridge tiles and inspect roof from roof level | High | 800 – 1,500 | Roofing Contractor |
| Chimney stack repointing | Medium | 400 – 700 | Masonry Contractor |
| Clear gutters: repair/replace downpipe | Medium | 200 – 400 | General Builder |
| Investigate damp at rear kitchen wall | High | TBC after survey | Damp Specialist |
| Confirm structural beam at chimney breast removal | High | 400 – 800 | Structural Engineer |
| Top up attic insulation to 270mm | Low | 600 – 1,200 | Insulation Contractor |
| Commission CCTV drainage survey | Medium | 250 – 500 | Drainage Contractor |
| Install CO detector | High | 30 – 60 | Homeowner/Electrician |
| Replace cracked drainage inspection chamber cover | Low | 80 – 150 | General Builder |
| Replace loose laminated flooring in sitting room | Low | 200 – 600 | Flooring Contractor |

8. Limitations & Exclusions

This inspection was conducted as a visual survey only. The following areas or elements were not inspected or tested and are outside the scope of this report:

- Areas concealed by furniture, floor coverings, wall linings, or stored items.

- Services and installations beyond visual observation are recommended.

- Areas below ground level, including foundations and underground drainage.

- Roof spaces where access was not available or safe to enter.

- Internal structure and fabric of walls, floors, and ceilings where finishes obscured them.

- Environmental hazards including radon, asbestos, lead paint, or ground contamination

- Legal matters including title, planning permission, or compliance certificates

9. Declaration & Certification

This report has been prepared solely for the use of the named client and must not be relied upon by any third party without the written consent of Viewpoint Ark Ltd.

This report does not constitute legal, financial, or structural engineering advice. Independent specialist reports are recommended where noted herein.

| Inspector Signature: Ciarán O’Brien | Company: Viewpoint Ark Ltd Website: www.viewpointark.com Report Reference: VPA-2025-0412 Issue Date: 7 April 2025 |
| --- | --- |